Codes

1. This code contains an adder task and a tester task that generates 10 random values and tests the adder task

module tb;

bit [3:0] a;

bit [3:0] b;

bit [3:0] res;

reg clk = 0;

int i;

//always #5 clk = ~clk;

task adder(input bit[3:0]a,input bit[3:0]b,output bit[3:0]res);

res = a+b;

endtask

task random_tester(input bit[3:0]a,input bit[3:0]b,output bit[3:0]res);

for(i=0;i<10;i++) begin

a = $urandom();

b = $urandom();

adder(a,b,res);

$display("A = %0d, B= %0d, Res = %0d",a,b,res);

end

endtask

initial begin

random_tester(a,b,res); end

endmodule

2. Swapping two numbers using pass by value method

module tb;

bit[2:0] a;

bit[2:0] b;

task swap(bit[2:0] a,b);

bit[2:0] temp;

temp = a;

a = b;

b = temp;

$display("A = %0d, B = %0d",a,b);

endtask

initial begin

a = 2;

b = 3;

swap(a,b);

$display("A = %0d, B = %0d",a,b);

end

endmodule

3. Swapping two numbers using pass by reference

module tb;

task automatic swap (ref int a, ref int b);

int temp;

temp = a;

a =b;

b = temp;

$display("%0d , %0d",a,b);

endtask

int a;

int b;

initial begin

a = 3;

b = 4;

swap(a,b);

$display("%0d , %0d",a,b);

end

endmodule

4.Creating custom constructor

class first;

bit[7:0] a;

bit[7:0] b;

bit[7:0] c;

function new(input bit[7:0] datain1=0,input bit[7:0] datain2=0,input bit[7:0] datain3=0);

a = datain1;

b = datain2;

c = datain3;

endfunction

endclass

module tb;

first f1;

initial begin

f1 = new(32,32,32);

$display("%0d  %0d   %0d",f1.a,f1.b,f1.c);

end

endmodule

5.Using class inside another class

- class first;

- int data = 34;

- task display();

- $display("Value of data : %0d",data);

- endtask

- endclass

- class second;

- first f1;

- function new();

- f1 = new();

- endfunction

- endclass

- module tb;

- second s;

- initial begin

- s = new();

- $display("Value of Data : %0d",s.f1.data);

- s.f1.display();

- s.f1.data = 45;

- s.f1.display();

- end

- Endmodule

6.Class - Generator

class generator;

rand bit[3:0]a,b;

bit[3:0] y;

endclass

module tb;

int i;

generator g1;

initial begin

g1 = new();

for(i=0;i<10;i++)

begin

g1.randomize();

$display("%0d , %0d ", g1.a,g1.b);

#10;

end

end

endmodule

7.Adding Constraints

class generator;

randc bit[3:0] a,b;

bit[3:0] y;

constraint data_a {a inside {[0:3],9};} /// Ranging from 0-3 includes 9

constraint data_b {b inside {[0:3],9};} /// Ranging from ""

endclass

module tb;

int i;

generator g1;

initial begin

for(i=0;i<10;i++) begin

g1 = new();

if(!g1.randomize()) begin

$display("Randomization Error, Exiting");

$finish();

end

$display("%0d  %0d",g1.a,g1.b);

end

end

Endmodule

8.Pre and Post randomisation and different bucket for RandC function.

- //pre-randomize

- //post-randomize

- class generator;

- randc bit [3:0] a,b;

- bit [3:0] y;

- int min;

- int max;

- function void pre_randomize(input int min, input int max);

- this.min = min;

- this.max = max;

- endfunction

- constraint data {

- a inside {[min:max]};

- b inside {[min:max]};

- }

- function void post_randomize();

- $display("Value of a :%0d and b: %0d", a,b);

- endfunction

- endclass

- module tb;

- int i =0;

- generator g;

- initial begin

- g = new();

- $display("SPACE 1");

- g.pre_randomize(3,12);

- for(i = 0; i<6;i++)begin

- g.randomize();

- #10;

- end

- $display("SPACE 2");

- g.pre_randomize(3,12);//3 4 5 6 7 8 9 10 11 12

- for(i = 0; i<6;i++)begin

- g.randomize();

- #10;

- end

- end

- Endmodule

9. := vs :/

- class first;

- rand bit wr; // :=

- rand bit rd; // :/

- rand bit [1:0] var1;

- rand bit [1:0] var2;

- constraint data {

- var1 dist {0 := 30, [1:3] := 90}; //0 = 30/300 , 1,2,3 = 90/300

- var2 dist {0  :/ 30 , [1:3] :/ 90}; //0,1,2,3, = 30/120 = 0.25

- }

- constraint cntrl {

- wr dist {0 := 30 , 1 := 70};

- rd dist {0 :/ 30 , 1 :/ 70};

- }

- endclass

- module tb;

- first f;

- initial begin

- f = new();

- for(int i = 0; i < 15; i++) begin

- f.randomize();

- $display("Value of Var1(:=) : %0d and Var2(:/) : %0d", f.var1, f.var2);

- end

- end

- endmodule

10.Implication Operator

class generator;

rand bit ce;

rand bit rst;

constraint control_ce{

ce dist {1:=80,0:=20};

}

constraint control_rst {

rst dist {0:=80,1:=20};

}

extern constraint contrl_rc;

endclass

constraint generator::contrl_rc

{

(rst==0) <-> (ce == 1); //// For all values of rst =0, ce will be 1

}

module tb;

int i;

generator g1;

initial begin

g1 = new();

for(i=0;i<10;i++) begin

g1.randomize();

$display("Values of rst & ce = %0d %0d",g1.rst,g1.ce);

end

end

endmodule

11.IF else Operator

- class generator;

- randc bit [3:0] raddr, waddr;

- rand bit wr; ///write to mem

- rand bit oe; ///output enable

- constraint wr_c {

- wr dist {0:= 50, 1 := 50};

- }

- constraint oe_c {

- oe dist {1:= 50, 0 := 50};

- }

- constraint wr_oe_c {

- ( wr == 1 ) <-> (oe == 0);

- }

- constraint write_read {

- if(wr == 1)

- {

- waddr inside {[11:15]};

- raddr == 0;

- }

- else

- {

- waddr == 0;

- raddr inside {[11:15]};

- }

- }

- endclass

- module tb;

- generator g;

- initial begin

- g = new();

- for (int i = 0; i<15 ; i++) begin

- assert(g.randomize()) else $display("Randomization Failed");

- $display("Value of wr : %0b | oe : %0b |  raddr : %0d | waddr : %0d |", g.wr, g.oe,g.raddr, g.waddr);

- end

- end

- endmodule

12.EVENT Triggering

- module tb;

- event a1,a2;

- initial begin

- ->a1;

- ->a2;

- end

- initial begin

- wait(a1.triggered);

- $display("Event A1 Trigger");

- wait(a2.triggered);

- $display("Event A2 Trigger");

- end

- endmodule

13.Mail Box Implementation

class generator;

int datag = 32;

mailbox mbx;

task run();

mbx.put(datag);

$display("Data Sent : %0d",datag);

endtask

endclass

class driver;

int datac =0;

mailbox mbx;

task run();

mbx.get(datac);

$display("Data Recieved : %0d",datac);

endtask

endclass

module tb;

mailbox mbx;

generator g;

driver d;

initial begin

mbx = new();

g = new();

d = new();

g.mbx = mbx; //// Linking mailbox

d.mbx = mbx;

g.run();

d.run();

end

endmodule

14. Mailbox demonstration between Generator and Driver with event trigger when done

class transaction;

rand bit [7:0] a;

rand bit [7:0] b;

rand bit wr;

endclass

class generator;

mailbox #(transaction) mbx;

transaction t;

event done;

function new(mailbox #(transaction) mbx);

this.mbx = mbx;

endfunction

int i;

task run();

for(i=0;i<10;i++) begin

t = new();

if(!t.randomize()) begin

$display("Failed at t = %0t",$time);

end

$display("Value generated are : %0d %0d %0d",t.a,t.b,t.wr);

mbx.put(t);

#10;

end

-> done;

endtask

endclass

class driver;

mailbox #(transaction) mbx;

transaction dc;

function new(mailbox #(transaction) mbx);

this.mbx = mbx;

endfunction

task run();

forever begin

mbx.get(dc);

$display("Data recieved %d %d %b", dc.a, dc.b, dc.wr);

end

endtask

endclass

module tb();

generator g;

driver d;

mailbox #(transaction) mbx;

initial begin

mbx = new();

g = new(mbx);

d = new(mbx);

fork

g.run();

d.run();

join

end

initial begin

wait(g.done.triggered);

$display("DONE");

end

endmodule